Lab 3 Report

Inverse Kinematics

Name: Ramiro Rojas

Section: 0

Date: 10/16/2026

# 1. Numerical IK equations

TODO 1  Position error

position = target[:3, 3] - current[:3, 3]

TODO 2  Damped least squares

step = jacobian.T @ np.linalg.solve(system, error)

TODO 3  Joint update

q_proposed = q + alpha * delta_q

# 2. Offline IK results

Target A converged: True reason: converged iterations: 14 final q [rad]: [ 0.19924 -0.7988 0.99875 -1.09713 -0.70111 0.29779] position error [mm]: 0.817 orientation error [deg]: 0.088 Target B converged: True reason: converged iterations: 17 final q [rad]: [-0.29899 -0.89984 1.09987 -1.39796 -1.20016 -0.1986 ] position error [mm]: 0.817 orientation error [deg]: 0.152

# 3. Target A Webots result

Target: A Desired tool position [m]: [0.662752 0.348085 0.277222] Desired tool RPY [rad]: [-2.561988 0.399181 -2.210128] Joint order: shoulder_pan_joint, shoulder_lift_joint, elbow_joint, wrist_1_joint, wrist_2_joint, wrist_3_joint Initial q [rad]: [-4.464087e-06 4.335639e-06 1.230183e-06 -3.028271e-07 1.902101e-08 -1.905286e-08] Converged: True Reason: converged Iterations: 20 Solver position error [mm]: 0.597 Solver orientation error [deg]: 0.402 IK solution q [rad]: [ 0.199736 -0.799209 0.996694 -1.091273 -0.698857 0.300827] Predicted tool position [m]: [0.662626 0.347941 0.277787] Predicted tool RPY [rad]: [-2.56739 0.394454 -2.213609] Webots measured q [rad]: [ 0.199736 -0.799163 0.996711 -1.091272 -0.698857 0.300827] Webots tool position [m]: [0.662309 0.348903 0.277716] Webots tool RPY [rad]: [-2.567442 0.394388 -2.213629]

Screenshot

Both the predicted and measured tool position and tool RPY for Target A agree closely, only differing by a small decimal value.

# 4. Target B Webots result

Target: B Desired tool position [m]: [ 0.726296 -0.047359 0.294847] Desired tool RPY [rad]: [-1.890963 0.41478 -1.809042] Joint order: shoulder_pan_joint, shoulder_lift_joint, elbow_joint, wrist_1_joint, wrist_2_joint, wrist_3_joint Initial q [rad]: [-4.464087e-06 4.335639e-06 1.230183e-06 -3.028271e-07 1.902101e-08 -1.905286e-08] Converged: True Reason: converged Iterations: 23 Solver position error [mm]: 0.440 Solver orientation error [deg]: 0.499 IK solution q [rad]: [-0.299917 -0.899023 1.096938 -1.390946 -1.198835 -0.196802] Predicted tool position [m]: [ 0.725994 -0.047091 0.295019] Predicted tool RPY [rad]: [-1.899832 0.41525 -1.815735] Webots measured q [rad]: [-0.299917 -0.898981 1.096955 -1.390944 -1.198835 -0.196802] Webots tool position [m]: [ 0.726195 -0.046269 0.294883] Webots tool RPY [rad]: [-1.899918 0.415246 -1.81577 ]

Screenshot

Both the predicted and measured tool position and tool RPY for Target B agree closely, only differing by a small decimal value.

# 5. Engineering question

During one IK iteration, what does the solver calculate by comparing the desired and current tool poses, and how does it use that result to update the joint angles?

During one IK iteration, the solver calculates the position error and orientation error by comparing the desired and current tool poses.